Expo Report Hazard Sensing Robot

Cheyane Nichols, Justin Nizolek, Kiden Scruton, Yousef Alshabbo

ES 143 - Faraci

12/3/2022

# Abstract

This project involves the design and demonstration of the Hazard Sensing Robot (HSR). The HSR uses multiple modules in order to complete its objective of bringing its user an overview of the environment it is in. The HSR uses a DHT-11 as the Temperature/ Humidity Sensor which gives a general overview of the dangers of the environment based off the temperature and humidity, a MQ-5 as the Gas sensor which tracks the composition of the gasses in the atmosphere around it along with the percentage composition of the gases in the air, and a Wi-Fi / Camera module that came with our kit that communicates live video and receives the movement commands with the remote user through a Wi-Fi based UART control method. In addition to the Wi-Fi camera module the HSR was outfitted with 2 vertical sets of led lighting to be better equipped for all environments. The HSR has a hard shell all around protecting the internal sensors and has a square wheelbase outfitted with grippy rubber tires also from the robot kits provided.

Table of Contents

Abstract	2

List of Figures & Tables	4

1.0 Introduction	5

2.0 Background Information	6

2.1 Context of the Design	6

2.2 Design Concept and Goals	6

2.3 Design Constraints	6

2.4 Design Proposals	6

2.5 Health & Safety/Environmental/Contemporary Issues	6

2.6 Design Plan	7

2.6.1 Summary of the Plan	7

2.6.2 Project Timeline	7

3.0 Results and Discussion	7

3.1 Demo Performance	7

3.2 Performance vs. Design Goals	7

3.3 Future Work	7

4.0 Summary and Recommendations	8

4.1 Project Summary	8

4.2 Acknowledgements	8

References	9

Appendix	10

# List of Figures & Tables

Figure 1: A picture	9

# 1.0 Introduction

A device called the Hazard Sensing Robot (HSR) was designed and demonstrated. It is a small compact robot that can go into spaces a person cannot go. It is designed to detect temperature and air quality in potentially hazardous environments. For example, during a fire the robot could detect the heat as well as the lack of carbon-dioxide (CO2) in the atmosphere. The H.S.R. robot is also Bluetooth operated so the camera can see exactly what the robot sees and the data from the DHT-11 Temp/Humidity sensor and the MQ-5 Gas sensor are recorded on an SD card, which is located on the robot and that can be accessed by the user after the expedition. The data can be plotted in a Microsoft Excel spreadsheet to show the change in values over time and show the dangers of the environment. H.S.R. uses two independent microcontrollers running two very different programs. The Arduino Uno is running a highly modified and cut down version of the code run on the “ELEGOO UNO R3 Project Smart Robot Car Kit V 3.0 Plus”. The code that is used is open source which allows free modification and distribution even commercially. The Seeed ino Xaio is running a code written by Kiden Scruton using past knowledge and some tutorials freely available on Arduino.org

# 2.0 Background Information

## 2.1 Context of the Design

(name, year of pub.)

The HSR robot is designed to go into urban scenarios where air quality is likely to be tainted by toxic gases such as carbon monoxide and propane. In addition to this, the HSR uses a temperature and humidity sensor to record the temperature of the environment it is in.  The robot does this to solve the problem in today's world in fields like search and rescue or responding to natural disasters in dangerous urban environments where you would not want to send in a person but a robot that is rugged, somewhat disposable and would cost less than 100$ would be perfectly suited for.

## 2.2 Design Concept and Goals

The goal of this design is to create a small compact rugged robot, that can perform various critical tasks in hazardous conditions such as urban areas affected by natural disasters or areas exposed to harmful chemicals and radiation.
The robot is designed to be mobile and rugged so its wheelbase is in the shape of a square so it is strong and can easily turn using a reliable tank style turning method. This also allows the robot to work on only 3 wheels and it will still be very capable. In order to make a rugged robot the frame cages around the sensors so nothing is damaged in case there is water splashed on it or it is exposed to sand and debris. Other ways we tried to make the robot as rugged as possible: Different tires, so the grip is increased. Bumpers around the camera so it is protected, and

## 2.3 Design Constraints

Getting sensors: shipping and money (gas sensor MQ5, tiny Arduino, air sensor, temperature sensor, ($30)

AutoCAD printing: time-consuming: whole robot frame 5-6 hours cage: 3 hours

Design around 4 days for initial design, 2 days for each prototype

Bluetooth: Some difficulties getting started

Manufacturing: Had to use materials that make an easily replicable robot as our robot should be able to be mass produced if we so desired and we will go though many design interations,  we also must use materials available at the school that is cheap to us students.

## 2.4 Design Proposals

Designs considered: Monster truck, tank, reasons for not doing these projects No purpose or task they would complete.

Started looking at other sensors available for temperature and air and brainstormed the idea of the robot detecting these “hazardous” things.

## 2.5 Health & Safety/Environmental/Contemporary Issues Health and safety: public safety, going into buildings, tight spaces, hazardous areas where people cannot go.

The Article below, I took a lot of inspiration from their designs. “SREHD is placed in a conditioning temperature under various extremes of temperature and humidity, then put through its paces on the range.” This is like what we are doing. A lot of these military-grade robots have a tank-like design. Initially one of our initial projects was a tank, however we decided with a time frame a compact robot would be best to print. (Ref #6) Our device is mostly made from PLA which is a type of plastic that can be printed by a 3D printer. Though PLA may not be a very environmentally friendly plastic, it is very cheap with 1 kilogram of it only being about 20$. As this is the majority of robot’s composition, we could easily change plastic to a much more environmentally friendly material. An example of a filament we could use for the robot is one made from plants “such as the soy based FilaSoy or SeaWeed, a material made from seaweed. The Algix from 3DFuel is also available and is made from nuisance algae.”  - (Ref #2) or we could use a filament made from recycled plastics like “Plastic waste extruders such as the Filabot or ProtoCycler can recycle this plastic waste. This innovation, with its ecological aspect, also represents a big cost saving for the users who can reduce their 3D filament expenses.” - (Ref #2) either of these options could give us a near carbon neutral effect on the environment due to plants taking carbon out of the atmosphere and into our robot. This could give us a very ecologically responsible robot that can easily be mass produced without much environmental impact.

## 2.6 Design Plan

To simplify the delegations of tasks based on the level of skill and experience in certain aspects of the project a chief designer was appointed, and a chief programmer was appointed. Then everyone else left in the group filled in to pick up where the chiefs fell short. This delegation of tasks allowed input from all members of the group but also allowed them to do what they were good at, and it really benefited the project. Then during the presentation and report times all members played an equal part in completing the presentations.

### 2.6.1 Summary of the Plan

Initial sketches: Included the chief designer to take input from all team

members and design a rough draft to base the project from

Research on sensors: Needed to figure out the spacing for the sensors and the actual data that could be received

frame prototypes 1-14 printed (Pictures of each)

Final fitting of sensors wires and frame

## 2.6.2 Project Timeline

2.6.2 Project timeline – Time/Task (Gantt Chart)

Date tracking Gantt chart1.xlsx

# 3.0 Results and Discussion

As a result, the HSR performed as expected during the expo presentation. The gas sensor never failed and was able to get consistent and reliable results every time. As for the temperature and humidity sensor, the same could be said about it and it never failed during the presentation. The Arduino and shield, however, was troublesome at times, often the Arduino shield would fail and leave the robot stranded. Also, it should be mentioned that Arduino too had trouble uploading/flashing at some points during the project. But those ended up to just be quality problems with the parts from the kits that the robot used

## 3.1 Demo Performance

The demo performance was perfect. Once the right components were sourced and tested the robots performed as they should have during the presentation. The HSR read and relayed back data perfectly, it was even equipped to write the data in a micro-SD card. The HSR could also drive over urban terrain and could be either user controlled or programmed to go through a specific route.

## 3.2 Performance vs. Design Goals

When the initial brainstorming was taking place, the original idea was to have the data transmit live feed back to the user’s computer. However, this wasn’t a feasible idea as the parts provided, their signal only reached a few feet. Another design goal that didn’t make it far off the drafting board was our idea to include water proofing to the robot, it turned out to be a large waste of time in the sense that there were other more important design features that needed to be taken care of first. After that, however all the goals the team had were incorporated successfully into the design.

3.3 Future Work

- Improvement of overall materials, possibly a metal frame. Upgrading the wheels to have more grip. Getting a better-quality camera. Blue tooth transmission was not done since it only went 30 feet out. Somehow getting a stronger blue tooth signal with more range like getting a higher end Bluetooth module. We would investigate installing an ultrasonic sensor to be able to navigate areas safely and potentially map out the terrain around it automatically. We would also likely scale the entire robot up as we had to make expansions near the end of the project timeline to fit all the components in it.

# 4.0 Summary and Recommendations

In summary, our robot was a huge learning experience. We had many setbacks that led us to learn from our mistakes or adapt towards a better way to move forward. I speak for all the group when I say this is a project, we are all immensely proud of it and it was a great group effort. There are things that could be recommended in the future, those being time management and quality of parts to name a few.

## 4.1 Project Summary

- In total our group even with the setback we occurred trough problems with SD card readers, Bluetooth and displays we still make it through and got the project completed in the end. We tied for 1st place at The Expo and showed off working prototypes of our H.S.R.. My group and I are very proud of ourselves and each other in the way we accomplished everything we set out to and had a very polished capable robot at the expo.

## 4.2 Acknowledgements

- We would like to thank Elegoo for their contribution to our code and their v4 robot kit, https://www.arduino.cc/ for their coding tutorials helping with the programming of the SD card on their website and Seeedino for their tutorials on their microcontrollers.

# References

Deslierres, Michel. “Table of Contents.” Overview of the SAMD21 Arm Cortex-M0+ Based Seeed Studio XIAO, https://sigmdel.ca/michel/ha/xiao/seeeduino_xiao_01_en.html#uart.

“Eco-Friendly 3D Filament Guide - What Are the Possibilities and Innovations.” Aniwaa, https://www.aniwaa.com/guide/am-materials/eco-friendly-3d-filament/.

“Elegoo Uno R3 Project Smart Robot Car Kit V 3.0 Plus.” ELEGOO Official, https://www.elegoo.com/products/elegoo-smart-robot-car-kit-v-3-0-plus.

“HC-05 (ZS-040) Bluetooth Module.” ProtoSupplies, 8 Oct. 2022, https://protosupplies.com/product/hc-05-zs-040-bluetooth-module/.

Person. “UART: A Hardware Communication Protocol Understanding Universal Asynchronous Receiver/Transmitter.” UART: A Hardware Communication Protocol Understanding Universal Asynchronous Receiver/Transmitter | Analog Devices, https://www.analog.com/en/analog-dialogue/articles/uart-a-hardware-communication-protocol.html.

“SREHDing Explosive Hazards: New Robotic System Finds and Destroys Explosive Threats.” Www.army.mil, https://www.army.mil/article/185466/srehding_explosive_hazards_new_robotic_system_finds_and_destroys_explosive_threats.

“TM1637 Tiny Display #.” TM1637TinyDisplay, https://jasonacox.github.io/TM1637TinyDisplay/.

Zuo, Baozhu. “Getting Started with Seeed Studio Xiao Samd21.” Seeedstudio, https://wiki.seeedstudio.com/Seeeduino-XIAO/.

name, a. (year of pub.).

# Appendix

Include material from outside sources used directly or indirectly in the study but not essential to include in the body of the study.

- Parts list

- Robot parts:

- 4 tires

- 4 motors

- Camera

- Mq-5 Gas sensor

- Temperature sensor

- Arduino seed

- 1 Arduino

- Top and Bottom Frame

- Cage

- Battery

- Bumper

- Motor covers

- Breadboard

- lightbar

- Wires

- Expo/6x6

- Extension cord

- Computer for data

- Poster

- Sand

- Gravel

- Wooden ramp

- Tarp

# Sd Card Expo code

#include <SPI.h>

#include <SD.h>S

#include <Arduino.h>

int T = 0;

int R = 4000; // HOW OFTEN TO READ SENSORS

int Seconds = 0;

#include "DHT.h"

#define DHTPIN 3

#define DHTTYPE DHT11

#define ledPin 1

#define sensorPin A0

DHT dht(DHTPIN, DHTTYPE);

File myFile;

void setup() {

dht.begin(); // initialize the sensor

pinMode(ledPin, OUTPUT);

digitalWrite(ledPin, LOW);

// Open serial communications and wait for port to open:

Serial.begin(9600);

while (!Serial) {

; // wait for serial port to connect. Needed for native USB port only

}

Serial.print("Initializing SD card...");

if (!SD.begin(10)) {

Serial.println("initialization failed!");

while (1);

}

Serial.println("initialization done.");

// open the file. note that only one file can be open at a time,

// so you have to close this one before opening another.

myFile = SD.open("STATS.txt", FILE_WRITE);

// if the file opened okay, write to it:

if (myFile) {

Serial.print("Writing to test.txt...");

myFile.println("This is a test file :)");

myFile.println("testing 1, 2, 3.");

for (int i = 0; i < 3; i++) {

myFile.println(i);

}

// close the file:

myFile.close();

Serial.println("done.");

} else {

// if the file didn't open, print an error:

Serial.println("error opening test.txt");

}

}

void loop() {

Seconds = millis()/1000;

T = T + 1;

// read humidity

float humi = dht.readHumidity();

// read temperature as Celsius

float tempC = dht.readTemperature();

// read temperature as Fahrenheit

float tempF = dht.readTemperature();

if (T >= R){

// check if any reads failed

if (isnan(humi) || isnan(tempC) || isnan(tempF)){

myFile.println("Failed to read from DHT sensor!");

} else{

myFile = SD.open("STATS.txt", FILE_WRITE);

myFile.print("Humidity: ");

myFile.print(humi);

myFile.print("%");

myFile.print(" | ");

myFile.print("Temperature: ");

myFile.print(tempC);

myFile.print("°C ~ ");

myFile.print(tempF);

myFile.println("°F");

myFile.print("Gas Sensor: ");

myFile.println(readSensor());

myFile.print("Time: ");

myFile.print(Seconds);

myFile.close();

Serial.print("Humidity: ");

Serial.print(humi);

Serial.print("%");

Serial.print(" | ");

Serial.print("Temperature: ");

Serial.print(tempC);

Serial.print("°C ~ ");

Serial.print(tempF);

Serial.println("°F");

Serial.print("Gas Sensor: ");

Serial.println(readSensor());

Serial.print("Time:");

Serial.print(Seconds);

}

T = 0;

}}

// This function returns the analog data to calling function

int readSensor() {

unsigned int sensorValue = analogRead(sensorPin); // Read the analog value from sensor

unsigned int outputValue = map(sensorValue, 0, 1023, 0, 255); // map the 10-bit data to 8-bit data

if (outputValue > 65)

analogWrite(ledPin, outputValue); // generate PWM signal

else

digitalWrite(ledPin, LOW

return outputValue; // Return analog moisture value

}

# Application Function Code

https://hartford0-my.sharepoint.com/:f:/g/personal/scruton_hartford_edu/ErwO-rGWf-hNoFFsA0pT2_gBMjZTTjzNrLIwrCOah_N1eQ?e=ArEN5z