ME 505 Modeling Report

Gyro-Scopic Balanced Inline Skateboard Project

Authors: Kiden Scruton, Wilonia Jerome

Course & Section: ME 505, Section 6:30–8:00 MW

Date: May 13 2025

2. Theory and Modeling

2.1 System Overview

Full System Description: Our mechatronic skateboard project uses an MPU-6050 IMU for roll/tilt sensing, an Arduino Uno for real-time control, a TB6600 stepper driver to command two contra-rotating flywheels, and manual potentiometer regulation for constant spin speed.

Electrical Interfaces & Wiring:

  - MPU-6050 → Arduino (I²C): SDA→A4, SCL→A5, VCC→5 V, GND→GND

  - TB6600 Logic Inputs: PUL+/DIR+/ENA+ tied to Arduino D8 (5 V), PUL–/DIR–/ENA– to GND

  - Power: 12 V battery → TB6600 VMOT and Arduino VIN; common ground

- Component Block Diagram: Sensor → Controller → Actuator → Mechanical Gyro → Feedback.

2.2 Component Theory

2.2.1 Sensing

- MPU-6050 Operation: 3-axis accelerometer + 3-axis gyro. We use a complementary filter:

- Baseline & Drift: ±3° deadzone, 0.2° drift over 5 min.

- We included a deadzone of +- 3 degrees for Prototype testing

2.2.2 Actuation

- Stepper Fundamentals: NEMA-17 with 60 oz-in holding torque, full-step resolution.

- TB6600 Interface: Pulse/direction/enable signals, microstep settings.

- Torque Test: Holding torque ~0.5 N·m at configured current.

- Final prototype required more holding torque than that decive was able to output – this prevented final testes of the prototype

2.2.3 Mechanical

- Gyroscopic Principle: Contra-rotating flywheels cancel net spin torque, leaving pure precession torque

- Mounting: 3D-printed gimbal adapters + radial bearings.

- MOI Calculations:

2.3 Overall System Model

Governing ODE:

(I_b)*​ϕ¨​+(2(I_s)​ω(K_d)​)ϕ˙​+(mgh+2(I_s)​ω(K_p)​)ϕ=0

(Townsend & Shenoi, “Gyrostabilizer Vehicular Technology”)

Parameter Definitions:

  - I_b: roll inertia of board

  - I_s: single flywheel inertia

  - ω: spin rate (rad/s)

  - K_p, K_d: P/D gains

  - mgh: gravitational restoring torque

Term Interpretation:

- I_b​  = MOI of the vehicle about the roll axis

- ϕ = Roll angle

- ϕ˙​ ,ϕ¨​  = Roll rate and roll acceleration

- I_s​  = Moment of inertia of one flywheel

- ω = Flywheel spin rate (rad/s)

- K_p​ , K_d​  = Control gains for position and velocity (PID-like gains)

- mgh = Gravitational restoring torque (mass × gravity × height of CG)

(I_b)*​ϕ¨ -> This represents the internal resistance to change calculated by the roll acceleration and the MOI about the roll axis ie roll inertia

(2(I_s)​ω(K_d)​)ϕ˙​ ->  This represents the damping  and prevents overshooting and oscillation. To increase or decrease the damping you change the var K_d

(mgh+2(I_s)​ω(K_p)​)ϕ -> This Represents the centering force AKA now hard the device attempts to right itself upwards. To change the strength, change the value of the var K_p. This eq also shows the deviation from the desired roll angle.

3. Simulation

3.1 Software Platform

- LabVIEW 2022 RT with Control Design & Simulation Module.

3.2 Block Diagram

3.3 Simulation Setup

- Initial Conditions:

- ϕ(0)=20°, ϕ˙(0)=0

Above shows an example of the underdamped inverted pendulum starting at 20 degrees

- Parameters: I_b=0.1022, I_s=0.003, ω=628,K_p=3,K_d=0.01,mgh=6.

- Solver: fixed-step Euler 4th-order Runge-Kutta , dt=0.01 s, total time=10 s.

3.4 Simulation Results

- Time-Series: ϕ(t) shows underdamped oscillation settling ~4 s.

- COG XY Plot: the underdamped pendulum is shown at angle ~-20 degrees after 10 seconds

- Phase-Plane: stable spiral toward origin.

4. Analysis

4.1 Simulation vs. Hardware

No hardware Test completed due to over torque issue although simulation tests have been done.

4.2 Discrepancies & Causes

When the simulation cannon maintain gyroscopic stability the simulation values increase to negative infinity – this is likely caused by a panic by the computer to compute what can be done to reorient the pendulum although given the constraints it is not possible and this simulation breaks; this is useful as it makes it clear which configurations are outside of the scope of practicality ex low rpm and high weight.

4.3 Recommendations for future improvement

- Model motor dynamics

- Introduce sensor latency/noise

- Add friction model

- Increase torque in stepper motors

- Increase stepper motor controller amperage

- Increase flywheel size

- PWM flywheel speed controller

5. Conclusion

- Objectives: Derived model, built sim & hardware, validated dynamics.

- Findings: Gains (Kp,Kd)~(3,0.01) yield stable underdamping and would be values to prove the usefulness through real-world tests

- Future Work: nonlinear model, full prototype test.

6. References

- Townsend, N., & Shenoi, R. (2011). Gyrostabilizer Vehicular Technology. ASME Applied Mechanics Reviews.

7. Appendix

- Full Arduino code

[Below] Arduino Code which results in the above CSV, Graph and was used to demonstraight our device -

#include <Wire.h>

#define MPU_ADDR    0x68

#define STEP_PIN    2

#define DIR_PIN     3

#define CTRL_PIN    7

#define VLOGIC_PIN  8   // 5V source for TB6600 logic+

// Constants

const float TILT_RANGE     = 80.0;

const float START_MOVE     = 5.0;

const float STOP_MOVE      = 2.0;

const int   MAX_STEPS      = 45;

const int   PULSE_WIDTH    = 100;

const int   MIN_DELAY      = 200;

const int   MAX_DELAY      = 3000;

const int   RETURN_DELAY   = 1000;

const float ALPHA          = 0.95;

long   currentStep     = 0;

float  zeroTilt        = 0;

float  filtTilt        = 0;

float  prevTilt        = 0;

float  angularAccel    = 0;

unsigned long lastTime = 0;

bool   moving          = false;

void setup() {

pinMode(STEP_PIN, OUTPUT);

pinMode(DIR_PIN, OUTPUT);

pinMode(CTRL_PIN, OUTPUT);

pinMode(VLOGIC_PIN, OUTPUT);

digitalWrite(STEP_PIN, LOW);

digitalWrite(CTRL_PIN, HIGH);      // ENA− active

digitalWrite(VLOGIC_PIN, HIGH);    // PUL+/DIR+/ENA+ logic HIGH

Serial.begin(115200);

Wire.begin();

// Wake MPU6050

Wire.beginTransmission(MPU_ADDR);

Wire.write(0x6B);

Wire.write(0x00);

Wire.endTransmission();

delay(200);

// Initial averaging

for (int i = 0; i < 10; i++) {

readRawAY();

delay(5);

}

zeroTilt     = getTilt();

prevTilt     = 0;

filtTilt     = 0;

currentStep  = 0;

moving       = false;

lastTime     = millis();

// CSV Header

Serial.println("Tilt,Steps");

}

void loop() {

float rawTilt = getTilt() - zeroTilt;

filtTilt = ALPHA * filtTilt + (1 - ALPHA) * rawTilt;

filtTilt = constrain(filtTilt, -TILT_RANGE, TILT_RANGE);

// Calculate angular acceleration

unsigned long now = millis();

float dt = (now - lastTime) / 1000.0;

float tiltVelocity = (filtTilt - prevTilt) / dt;

angularAccel = (tiltVelocity - (prevTilt - zeroTilt) / dt) / dt;

prevTilt = filtTilt;

lastTime = now;

// 📟 CSV Output Only

Serial.print(filtTilt, 2);

Serial.print(",");

Serial.println(currentStep);

// Hysteresis for motion logic

if (!moving && fabs(filtTilt) > START_MOVE) moving = true;

else if (moving && fabs(filtTilt) < STOP_MOVE) moving = false;

// Movement execution

if (moving) {

bool forward = (filtTilt > 0);

if ((forward && currentStep < MAX_STEPS) ||

(!forward && currentStep > -MAX_STEPS)) {

digitalWrite(DIR_PIN, forward ? HIGH : LOW);

float accelFactor = constrain(fabs(angularAccel) / 50.0, 0.0, 1.0);

float scaled = accelFactor * accelFactor;

int delayTime = map(int(scaled * 1000), 0, 1000, MAX_DELAY, MIN_DELAY);

stepPulse(delayTime);

currentStep += forward ? 1 : -1;

}

return;

}

// Return to center logic

if (abs(currentStep) > 0 && fabs(filtTilt) < STOP_MOVE) {

digitalWrite(DIR_PIN, currentStep < 0 ? HIGH : LOW);

stepPulse(RETURN_DELAY);

currentStep += (currentStep < 0) ? 1 : -1;

}

}

void stepPulse(int delayTime) {

digitalWrite(STEP_PIN, HIGH);

delayMicroseconds(PULSE_WIDTH);

digitalWrite(STEP_PIN, LOW);

delayMicroseconds(delayTime);

}

float getTilt() {

int16_t ay = readRawAY();

return map(ay, -16000, 16000, -90, 90);

}

int16_t readRawAY() {

Wire.beginTransmission(MPU_ADDR);

Wire.write(0x3D); // ACCEL_YOUT_H

Wire.endTransmission(false);

Wire.requestFrom(MPU_ADDR, 2, true);

if (Wire.available() < 2) return 0;

return (Wire.read() << 8) | Wire.read();

}

LabVIEW VI block diagram